Guide to Notes:

Overview: Multi-step and distributed processing using quasi-observations	2

Automatic processing with sh_gamit and sh_glred	3

Evaluating results	4

process.defaults	5

site.defaults	7

sestbl. and  sittbl.	8

autcln.cmd	9

rcvant.dat and hi.dat	11

Antenna effects	12

Tropospheric effects	13

sh_metutils	14

Using GAMIT effectively (pitfalls and refinements)	15

Using GLOBK effectively (subtleties and pitfalls) : what globk does	16

What glorg does	17

globk/glorg:  parameterization and controls	18

globk/glorg command file templates	20

Using glist	25

Defining the reference frame	26

Analysis example with earthquakes : S.California 1992-1999	27

Error analysis and data weighting	30

Abbreviations for References:

IntroGG : Introduction to GAMIT/GLOBK

GAMIT  : GAMIT Reference Manual

GLOBK  : GLOBK Reference Manual

All available at http://www-gpsg.mit.edu/~simon/gtgk/docs.htm

Overview:  Multi-step and distributed processing using quasi-observations

GAMIT  (each network each day)

Primary observations = phase

Estimated parameters = station coordinates; zenith delays, gradients, 					ambiguities, EOP, orbital parameters

Output = h-file of parameter estimates and covariances—loosely constrained

GLOBK  (combination of networks and days)

Quasi-observations = multiple h-files of coordinates, EOP

orbital parameters (or SINEX without orbital parameters)

Estimated parameters = coordinates and velocities, daily or multiday orbits

Constraints = finite or generalized

Output = time series   or

combined h-file; tables of positions and/or velocities for plotting

References:  IntroGG, Chapter 1;  GLOBK, Chapter 1

Dong et al. [J. Geod. 72. 200–214, 1998]

# Automatic processing with sh_gamit and sh_glred

Setup

Make experiment directory; sh_setup; copy RINEX files

Commands

sh_gamit –s 1998 151 154 –expt emed –orbit IGSF  -pres

sh_glred –s 1998 151 1998 154 –expt emed –ncomb 1 –opt H G E

Data files to be accessible

raw and/or RINEX

local or ftp access to orbits (sp3 or g-files), nav message, and EOP

reasonable starting coordinates (apr, l-file, or sh_rx2apr from RINEX)

rcvr/antenna info (station.info or RINEX header)

Control files to be edited

process.defaults: computer environment, source of files, session definition

sites.defaults	: global RINEX to get

sestbl.	: how to process (e.g. iterations, weighting, tropo)

sittbl.	: coordinate constraints, antenna phase-center model, etc.

station.info	: rcvr/ant/firmware, HI (optional if RINEX header correct)

Processing outputs

tabular summary of station statistics (autcln.post.sum)

sky plots of phase residuals for each station (Figure 1)

loosely constrained solution for each day (h-file)

time series of station coordinates

Reference:  IntroGG

Directory Structure

gg   /my_gamit-globk

/gamit		/kf	/bin	   /tables	/templates

/emed98 (e.g.)

/tables	(create from gg/tables using sh_setup)

/rinex	(observation files)

/brdc	(navigation files)

/igs	(sp3 orbit files)

/gfiles

/151	(day directories)

/152

…

/gifs	(gif format of sky plots)

/gsoln	(sh_glred or GLOBK runs)

Evaluating results

E-mail summary from sh_gamit

Number of stations

Worst/best rms of phase residuals

SOLVE postfit normalized RMS ~0.2

Large adjustments to station coordinates

Any FATAL messages

Input options -expt pcnw -d 1998 243 245 246 -orbit IGSF -pres Y -copt o q mkx -dopt D ao b c g i j k m p ps t x y -sessinfo 30 720 18 00

Processing 1998 245 GPS week 0973 3 Raw 8

/data53/rwk/oregon/pnw98/245

Disk Usage:  73094.6  Free  33857.8 Mbyte. Used 69%

Number of stations used 41 Total xfiles 41

Postfit RMS rms, to and by satellite

RMS  IT Site   All  01  02  03  04  05  06  07  08  09  10  13  14  15 . . .

RMS  26 ALL    6.8   6   0   6  11   0   0   7   8  10   0   0   7   6 . . .

Best and Worst two sites:

RMS  26 NEWP   0.0   0   0   0   0   0   0   0   0   0   0   0   0   0 . . .

RMS  26 BROT   4.5   4   0   5   0   0   0   0   4   6   0   0   5   4 . . .

RMS  26 WHD1   9.9   8   0   8  17   0   0   0  11  13   0   0   9  10 . . .

RMS  26 CMBB  10.5  10   0   9   0   0   0   0  13  14   0   0  10   8 . . .

Double difference statistics

Prefit nrms:  0.12403E+03    Postfit nrms: 0.17807E+00

Prefit nrms:  0.12341E+03    Postfit nrms: 0.19016E+00

Prefit nrms:  0.12380E+03    Postfit nrms: 0.17585E+00

Prefit nrms:  0.12341E+03    Postfit nrms: 0.18155E+00

Number of double differences:  36659

List of sites updated in the tables lfile from adjustments in the Prefit Solution

SKYO RADIUS     -0.4635

List of sites without Prefit coordinate solutions:

NEWP

List of sites without Postfit coordinate solutions:

NEWP

Detailed evaluation

WARNING messages

AUTCLN summary file:  RMS, data amounts, bias flags [GAMIT 4.2]

Adjustments to station coordinates, atmospheric parameters

Ambiguity resolution [GAMIT 3.4]

Repeatability plots:  scatter, uncertainties

Process.defaults

#  Do not remove any of these entries.

#To by-pass a function, set the value to null: ""

## LOCAL DIRECTORIES

# Directory for translation of raw data

set rawpth = "$procdir/raw"

# Directory path for raw archives (search all levels); e.g. /data18/simon

set rawfnd = ""

# Input files for RINEX translators

set mpth = "$procdir/mkrinex"

# RINEX files directory

set rpth = "$procdir/rinex"

# Dir path for RINEX archives (search all levels); e.g. /data18/simon

set rnxfnd = ""

# Broadcast orbit directory

set bpth = "$procdir/brdc"

# IGS files directory

set ipth = "$procdir/igs"

# G-files directory

set gpth = "$procdir/gfiles"

# GAMIT and GLOBK tables directory

set tpth = "$procdir/tables"

# Output gifs directory

set gifpth = "$procdir/gifs"

# Globk solution directory

set glbpth = "$procdir/gsoln"

# Globk binary h-file directory

set glfpth = "$procdir/glbf"

# Dir path for other h-files (search all levels); e.g. /raid1/tah/SIO_GLL

set hfnd = ""

# Template files

set templatepth = "$procdir/templates"

# Place to store temporary control files

set cpth = "$procdir/control"

# Archive root directory (cannot be null)

set archivepth = "$procdir/archive"

## FTP INFO FOR REMOTE FILES

# Raw data archive

# set rawarchive = 'chandler.mit.edu'

# set rawdir = 'pub/continuous/mitnet'

# set rawlogin = "anonymous simon@chandler.mit.edu"

# Addresses for CDDSI, SOPAC, IGSCB, UNAVCO, BKG, IGN, USNO are

#    given in template/ftp_info

##GAMIT

# Set sampling interval, number of epochs, and start time for processing

set sint = '30'

set nepc = '2880'

set stime = '0 0'

# Variables for updating tables

set stinf_unique = "-u"

set stinf_nosort = "-nosort"

set stinf_slthgt = "2.00"

# Set "Y" to use RINEX header coordinates not in lfile or apr file

set use_rxc      = "N"

# Broadcast orbits

set brdc = 'brdc'

# Minimum x-file size to be used (Default 300 blocks)

set minxf = '300'

# Set search window for RINEX files which might contain data for day

#    --default checkthe previous day

set rx_doy_plus = 0

set rx_doy_minus = 1

# Default globk .apr file

set aprf = itrf00.apr

# Set compress (copts), delete (dopts) and archive (aopts) options.

# Possible d-, c-, and a- opts:

# all D, H, ao, ac, as, b, c, d, e, g, h, i, j, k, l, m, o, p, q, t, x, ps

set dopts = ( c )

set copts = ( x k ao )

set aopts = ''

# Set the ftp archives (defined in ftp_info) to search for data.

# (Default archives searched are: sopac, cddis and unavco).

set rinex_ftpsites = (sopac cddis unavco)

## RESOURCES

# Minimum raw disk space in Kbytes

set minraw = '30000'

# Minimum rinex disk space in Kbytes

set minrinex = '30000'

# Minimum archive disk space in Kbytes

set minarchive = '20000'

# Minimum working disk space in Kbytes

set minwork = '200000'

## SYSTEM-DEPENDENT SETTINGS

# UNIX df command must be set to return the correct form

# Most machines

set udf = 'df -k'

# HP

# set udf = 'bdf'

# UNIX mail command

#  Most machines

set umail = 'mail -s'

#  HP

# set umail = 'mailx -s'

# Ghostscript path

set gspath = '/usr/bin'

# ImageMagick path fir gif conversion

# set impath = '/usr/bin/X11'

set impath = '/usr/bin'

## Web interface .html file.

set htmlinfo = 'mitnet.html'

## USER ADDRESSES

# Mail address for report (if '' will default to `whoami` in sh_gamit)

set mailto = 'rwk'

# Host name for email and anonymous ftp password use

#   (if '' will default to `hostname` in sh_gamit)

set machine = ''

Sites.defaults

# File to control the use of stations in the processing

#

# Format: site expt keyword1 keyword2 ....

#

#  where the first token is site name (GAMIT uses only 4 characters,

#    GLOBK 4 or 8),

#  the second token is the 4-char experiment name, and the remaining

#  tokens, read free-format, indicate how the site is to be used

#  All sites for which there are RINEX files in the local directory will

#  be used automatically and do not need to be listed.

#

# GAMIT:

#  ftprnx = sites to ftp from rinex data archives.

#  ftpraw = sites to ftp from raw data archives.

#  localrx = site names used to search for rinex files on local system.

#            (required in conjunction with rnxfnd path variable set in

#                process.defaults).

#  xstinfo = sites to exclude from automatic station.info updating.

#  xsite   = sites to exclude from processing, all or specified days

# GLOBK:

#  glrepu = sites used in the GLRED repeatabilities (default is use all)

#  glreps = sites used for reference frame definition (stabilization) in

#           GLORG for GLRED repeatability solution (default is IGS list)

#  glts   = sites to plot as time series (default is all)

#

# may use the following

# all_sites tubi xstinfo

# individual station commands will override all_sites if they follow it

mate_gps  tubi ftprnx xstinfo glrepu glreps

ankr_gps  tubi ftprnx xstinfo glrepu glreps glts

kit3_gps  tubi ftprnx xstinfo glrepu glreps

nssp_gps  tubi ftprnx xstinfo glrepu glreps

graz_gps  tubi ftprnx xstinfo glrepu glreps

noto_gps  tubi ftprnx xstinfo glrepu glreps

nico_gps  tubi ftprnx xstinfo glrepu

sofi_gps  tubi ftprnx xstinfo glrepu

zeck_gps  tubi ftprnx xstinfo glrepu

zwen_gps  tubi ftprnx xstinfo glrepu glreps

bahr_gps  tubi ftprnx xstinfo glrepu

tela_gps  tubi ftprnx xstinfo glrepu

tubi_gps  tubi ftpraw xstinfo glrepu        glts

mert_gps  tubi ftpraw         glrepu        glts

kant_gps  tubi ftpraw         glrepu        glts

ttth_gps  tubi xstinfo xsite:1999_256-1999_278

thht_gps  tubi xstinfo xsite glreps

Sestbl.

Processing Agency = MIT

Satellite Constraint = Y   ; Y/N  (next 2 lines free-format but 'all' must be present)

all   a   e   i   n   w   M  rad1  rad2  rad3  rad4  rad5  rad6  rad7  rad8  rad9;

.01 .01 .01 .01 .01 .01  .001  .001  .001  .001  .001  .001  .001  .001  .001

Choice of Experiment = RELAX.   ; BASELINE/RELAX./ORBIT

Type of Analysis = 1-ITER       ; 1-ITER/0-ITER (no postfit autcln)/PREFIT

This now redundant AUTCLN Postfit = R  ; Postfit autcln (Y or R for repeat)

Choice of Observable = LC_AUTCLN ; L1_SINGLE/L1&L2/L1_ONLY/L2_ONLY/LC_ONLY/

; L1,L2_INDEPEND./LC_HELP/LC_AUTCLN

Quick-pre observable = LC_ONLY    ; For 1st solution (default same as final

Station Error = ELEVATION 10 5   ; 1-way L1, a**2 + (b**2)(L**2) in mm, ppm

Use N-file = Y                  ; Y/N (default No): automatic reweight by station rms

Met obs source = STP 50         ; hierarchical list:  RNX ufile GPT/STP [humid value]

' to match 10.2, use STP 50; new default is GTP 50

Output met = N                  ; write the a priori met values to a z-file (Y/N)

Use met.list = N

Use met.grid = N

DMap = VMF1                   ; GMF(default)/NMFH/VMF1

WMap = VMF1                   ; GMF(default)/NMFW/VMF1

Use map.list = N

Use map.grid = N

Zenith Delay Estimation = Y   ; Y/N  (default No)

Atmospheric gradients = Y     ; Y/N  (default No)

Gradient Constraints = 0.01   ; gradient at 10 deg elevation in meters; default 0.03 m

Interval Zen = 2              ; 2 hrs = 13 knots per day

Zenith Constraints = 0.5      ; zenith-delay a priori constraint in meters (default 0.5)

Zenith Variation = 0.02 100.  ; zenith-delay variation, tau in meters/sqrt(hr), hrs

Ambiguity resolution WL = 0.15 0.15 1000. 10. 500. ; for LC_HELP only

Ionospheric Constraints =  0.0 mm + 8.00 ppm  ; for LC_HELP, ok for mid-solar-cycle

Ambiguity resolution NL = 0.15 0.15 1000. 10. 500.  ; narrow-lane deviation, sigma (cyc)

Elevation Cutoff = 0             ; default 0 to use value in autcln.cmd

Wobble Constraint = 0.003 0.0001 ; 3 mas .1 mas/day for small networks

UT1 Constraint = 0.00002 0.0001  ; .02 ms .1 ms/day for small networks

Tides applied = 31             ;  Binary coded: 1 earth  2 freq-dep  4 pole  8 ocean

16 remove mean for pole tide  32 atmosphere

Use otl.list = N

Use otl.grid = Y

Use atl.list = N

Use atl.grid = N

Etide model = IERS03              ; IERS96/IERS03

Apply atm loading = N             ; Y/N for atmospheric loading

Use atml.list = N

Use atml.grid = N

Antenna Model = ELEV              ; NONE/ELEV/AZEL  default = ELEV

SV antenna model = ELEV           ; NONE/ELEV       default = NONE

Update tolerance = 0.3            ; minimum adj. for updating L-file coords (def 0.3)

SOLVE print  = Y                  ; Turn on SOLVE output to screen (default N)

Scratch directory = /tmp

File:  sittbl.

SITE              FIX    --COORD.CONSTR.--

<< default for regional stations >>

ALL               NNN    100.  100.   100.

<< IGS core stations >>

VILL VILL_GPS     NNN    0.005 0.005  0.01

MAS1 MAS1_GPS     NNN    0.005 0.005  0.01

FORT FORT_GPS     NNN    0.005 0.005  0.01

File:  autcln.cmd

* Command file for AUTCLN version 3.125 to be used for global and regional data

* Default values are listed with comment flag (non-blank first character)

* Last edited by tah/rwk/scm  030415

* Don't use any GAMIT edits

use_gamit  no

* Remove more bias flags by allowing a base satellite if multiple slips

allow_one_bg  yes

* Allow extra missing epochs before flagging a data point if there are frequent

* dropouts in telemetry

*  gap_size all 3

* Set minimum elevation for editing and output: 15 10 better for older receivers

*  also sets minimum SNR

site_param all 10 10 0 0

* Set the ionospheric tolerances so you don't throw out too much data.

* These are the current defaults and will work under both low and high

* ionospheric conditions with well-behaved receivers.  For poorly tracking

* receivers and low ionosphere, you can improve the editing using

* 240 4 0.3 0.8.

*  ion_jump all   30  6 2 5

* Criteria for detecting slips (initial bias flags).  Defaults shown.

* First three are for WL, irrelevant for codeless L2 receivers

* Second three (LC) might be set tighter (e.g. 4 0.2 0.5) to catch

* partial-cycle jumps with poorly performing receivers.

* With poor prefit coordinates, set the last two numbers to 2 5 (or 5 10)

* but use the defaults for POST or, with noisy data, skip the postfit

* edit until a second pass with good coordinates allows tight detection of jumps.

*  dd_fit_tol 5 2 10   3 0.35 0.8

* The following three commands control the repair of cycle slips and subsequent

* removed of bias flags. The default values are conservative in the sense

* that they retain the most data.  They are optimal for global networks but

* will work ok also for regional networks.  However, for better ambiguity

* resolution in regional networks, different values are optimal.

*

* Set the tolerances used in trimming the one-way data to remove small

* segments between bias flags.  The following are defaults:

*  trim_oneway   120  8  0.1  24

* For regional networks use

trim_oneway   1000  10  0.2  50

* The first two parameters are the minimum times in seconds and minimum

* epochs for attempting to remove a bias flag; the last two are the minimum

* fraction of total span and minimum number of epochs allowed after last bias

* flag.  To strengthen ambiguity resolution for regional data, increase the

* last two parameters.  For fewer bias flags in 24-hr data increase the first

* two parameters.

*

* Number of data used to repair cycle slips.  Defaults are ok for all data but

* all values could be reduced for data sampled less often than 30s.

*  dd_return_size  100  50  10  10

*

* DD criteria for removing bias flags: chi-sq ratio  chi-sq min  max gap  gap scale

* For global networks use

*  remove_bias 10  3  1800  5

* For regional networks use

remove_bias 10  3  3600  5

* For fewer flags but more risk over small gaps, decrease the first value (see

* autcln.out).  For fewer flags and more risk over large gaps, increase the

* third and decrease the fourth,

* Maximum number of bias flags per SV before deleting all the data.

* Default infinite (not checked).

max_scan_edit  30

* To enhance numerical stability in SOLVE (but be careful in interpreting

* one-way residuals)

apply_phs_clk 1

* Set the summary filename to agree with the command file produced by FIXDRV

summary autcln.prefit.sum

* Commands to be used if post-fit editing invoked in the sestbl.

POST  summary  autcln.post.sum

POST  apply_phs_clk 30

POST  use_postfit

POST  postfit_edit 10 4.0

* Remove biases in one-ways after postfit edit

POST pf_remove_bf

* Possibly allow patching over larger gaps

* POST  remove_bias 10  3  3600 2

* Output phase residuals for sky plots

POST  phs_res_root DPH

* Resolve widelane ambiguities in autcln

POST lc_autcln

* Explicit edits added by sh_autedit or the analyst

x edit_site_sv algo 0 1 2800

x edit_site_sv all 23 1 400

x edit_site_sv trom 15 451 460

...

File: rcvant.dat

# Non-blank first column indicates a comment.

# An asterisk in column 14 indicates that a receiver or antenna is currently supported

# The (new) flags in column 37 for receivers indicate how differential code biases are to

# be treated:

#  'P' : receiver is cross-correlating and requires correction of P2' and C1

#          Rogue SNR, Trimble 4000, etc.

#  'C' : receiver is non-cross-correlating but reports C1 instead of P1

#         Trimble 4700, 5700, Leica RS500, CRS1000, SR9600, etc. unless AS is off

#  'N' : receiver is non-cross-correlating and reports true P1, P2

# Blank indicates that we have not determined the correct flag and GAMIT will stop until

# this file is modified to reflect the correct description of the receiver.

#

RECEIVERS

#

#RECCOD        IGS 20-char code       | Description (comment field)

#format(1x,a6,8x,a20)                 |

#

# Trimble receivers

TRMSST      * TRIMBLE 4000SST      P |  Dual freq. L1 C/A; L2 squaring; L2 P-code optional

TR5700      * TRIMBLE 5700         C |  L1/L2 receiver with Maxwell-4 ASIC

TRNTRS      * TRIMBLE NETRS        N |  L1/L2+L2C with Maxwell-5 ASIC, ethernet, ext freq.

# Ashtech receivers

ATMCRZ        ASHTECH MICROZ       N |  12  channel, L1/L2 P1/P2 (MicroZ)

ATZ12T      * ASHTECH Z-XII3T      N |  12  channel, L1/L2 P1/P2 with Metronome option (note firmware reports "Z-XII3")

ATZ12G        ASHTECH Z-XII3GETT   N |  12  channel, L1/L2 P1/P2 modified for time transfer (note: firmware reports "Z-XII3")

END

#

#

ANTENNAS

##ANTCOD        ____FULL_NAME_______ |

#format(1x,a6,8x,a15,1x,A4)         |

# Trimble antennas

TRMSSE      * TRM22020.00+GP       |  Geod. L1/L2 compact, with groundplane, Model 22020-00

TRMZEP      * TRM39105.00          |  Zephyr 4-point feed antenna - No Ground Plane

TRMZGP      * TRM41249.00          |  Zephyr 4-point feed antenna - Stealth Ground Plane

END

File: hi.dat

# File: hi.dat  Format version 1.0  (keep the version number in columns 32-24 of line 1)

# Table of mechanical offsets between measured height-of-instrument (HI) and the antenna # reference point (ARP)

# Entries last changed by rwk 060727

#

# Non-blank first column is a comment

# Comments can follow within the line (unread)

#

# Some antennas have equivalences in gamit/lib/hisub2.f and are given here only once.

# If the antenna you want is not in the equivalence list, you can add it to the table.

# A warning will be issued for entries missing from the table, and offsets set = 0.

# station.info entries of L1PHC or UNKNWN DHARP are both given 0. offsets with no warning.

#

# Format (1x,a20,1x,a6,1x,f6.4,1x,f6.4)

# 20-char RINEX code  6-char GAMIT code   5-char HI code  vert offset (m)   horiz offset (m)

JPLD/M_R             ROGSNR DHARP 0.     0.     ! ARP is bottom of choke ring / base of antenna

JPLD/M_R             ROGSNR DHBCR 0.     0.     ! direct ht to bottom of choke ring

JPLD/M_R             ROGSNR DHTCR 0.0699 0.     ! direct ht to top of choke ring (JPL)

JPLD/M_R             ROGSNR SLBCR 0.0699 0.1905 ! slant height to bottom edge of choke ring

Antenna Effects

Antenna-specific effects

Ground-antenna phase-center variations (PCVs)

IntroGG 1.2;   file antmod.dat

Mader, G.L., NGS: http://www.ngs.noaa.gov/ANTCAL/Files/summary.html

Site-specific effects

Elosequi et al., J. Geophys. Res., 100, 9921, 1995.

Satellite antenna effects

Schmid and Rothacher, “Estimation of elevation-dependent satellite antenna

phase center variations of GPS satellites, J. Geod., 77, 440, 2003.

# Tropospheric Effects

Errors in geodetic measurements

mapping functions in hydrostatic (“dry”) delay

[http://gauss.gge.unb.ca/papers.pdf/igs97tropo.pdf ]

gradients [Chen and Herring, J.Geophys. Res., 102, 20489, 1997; GAMIT Chap. 7]

parameterizing the zenith delay for estimation

Tutorial on all aspects:  http://bowie.mit.edu/~tah/12.540/12.540.Lec15.html

Extracting precipitable water from GPS measurements

model correctly the “dry” delay (pressure measurements, mapping function)

zenith wet delay

z-files, sh_metutil

tomography

Tutorial and research results: http://www.gpsmet.noaa.gov

(click on 2000 and 2002 Techincal Reviews_)

File:  sh_metutils

sh_metutil calls gamit/utils/metutil to read estimated zenith delays from a GAMIT o-file or

a SINEX zpd file and compute zenith wet delay (ZWD) and precipitable water (PW) using

the zenith hydrostatic delay (ZHD) computed using pressure values from a RINEX met file

or a GAMIT z-file.

Usage: sh_metutil -f [ZTD file] -m [RINEX met files] -z [z-file] -i [interval

where  [ZTD file] is the name of the SOLVE -file ( o[expt]a.ddd )

[RINEX met files ] gives the list of RINEX met files to be used (wildcards ok)  )

[z-file] gives the list of z-files to be read (wildcards ok)

[interval] is the epoch interval of the output file

E  : use the interval of the ZTD file values (default)

O  : use the interval of the z-file (GAMIT observations0

nn  : value in seconds of the interval

Notes: (1) ZTD file is required

(2) Either a RINEX met file or a GAMIT z-file is required

(for now, you cannot have both)

(3) The start time will always be set by the ZTD file

Examples: sh_metutil  -f oeuraa.223  -z zkosg4.223

sh_metutil  -f oeuraa.223  -m *2230.04m  -s 1.6

sh_metutil  -f oeuraa.223  -z z*4.223 -i O

Output files are named met_ssss.yyddd where ssss is the site name from the

z-files or RINEX met files, yy is the 2-digit year, and ddd is the day-of-year

-----------------------------------------------------------------------------------

File: otesta.220  (GAMIT o-file from SOLVE; machine readable version of q-file)

(Adj m       Sigma     Estimate)

ATM_ZEN X KAZA  2 2000  8  7  0  0  0.0363 +-   0.0126    2.1079

ATM_ZEN X KAZA  2 2000  8  7  2  0  0.0294 +-   0.0084    2.1010

ATM_ZEN X KAZA  2 2000  8  7  4  0  0.0170 +-   0.0078    2.0886

...

ATM_ZEN X KIT3  3 2000  8  7  0  0 -0.0082 +-   0.0154    2.2540

ATM_ZEN X KIT3  3 2000  8  7  2  0 -0.0054 +-   0.0106    2.2568

ATM_ZEN X KIT3  3 2000  8  7  4  0 -0.0118 +-   0.0099    2.2504

...

--------------------------------------------------------------------------------------------

File:  zkaza0.220  (GAMIT z-file from MODEL)

* A priori atmospheric values for KAZA

* Models:  Dry Zen SAAS  Wet Zen SAAS  Dry Map NMFH  Wet Map NMFW

* Yr  Doy Hr Mn Sec  PRN  Azimuth  Elevation  Pres   Temp    WV Pres

2000 220  0  0 30.   4   47.2360  52.2177     858.5  293.2   11.7

2000 220  0  0 30.  24    4.8970  85.9935     858.5  293.2   11.7

2000 220  0  0 30.  30  310.4261  32.4934     858.5  293.2   11.7

2000 220  0  1  0.   4   47.2447  51.9959     858.5  293.2   11.7

Dry Zen Wet Zen  Total Zen  Dry Map   Wet Map  Dry Slant  Wet Slant Total Slant

1.9562  0.1154     2.0716   1.2643    1.2648    2.4733     0.1460     2.6193

1.9562  0.1154     2.0716   1.0024    1.0024    1.9610     0.1157     2.0767

1.9562  0.1154     2.0716   1.8559    1.8589    3.6305     0.2145     3.8451

1.9562  0.1154     2.0716   1.2681    1.2686    2.4808     0.1464     2.6272

Using GAMIT effectively (pitfalls and refinements)

Setup

process.defaults  [IntroGG  2.3]

sites.defaults  [IntrodGG 2.3]

station.info  [IntroGG 2.1; GAMIT 2.3

a priori coordinates  [IntroGG 2.1; GAMIT  2.2]

network splits  [GAMIT  2.3]

session splits

makexp messages (missing or misnamed files)

makex  messages (DATA REJECTED; GAMIT 2.8]

fixdrv messages (clock behavior; GAMIT 2.10, 3.2]

model messages  (RCLOCK)

Problems with autcln

# 1 :  poor a priori coordinates

interpreting autcln.post.sum

Modeling and recovering the troposphere

elevation-dependent weighting (n-files)

piecewise-linear model for zenith delay and gradients

recovering precipitable water vapor

Ambiguity resolution

widelanes with pseudorange in autcln

ionospheric constraint [Dong and Bock, J. Geophys. Res., 94, 3949, 1989; GAMIT 3.4

tolerances for deviation from an integer

constraints for the "tight" solution

Using GLOBK effectively (subtleties and pitfalls)

# What globk does

Initialization ( glinit  )

decode the command file (com_file, srt_file, lst_file, eq_file, make_svs first)

read headers of all h-files and sort by time

report any (GAMIT) model differences among h-files (EOP, tides)

make a file of a priori satellite parameters (svs_file)

read the eq_file and generate names of stations

initialize apriori values of parameters from the h-file estimates

based on use_site/use_pos/use_num commands, make a list of stations to be used

read apr_file and replace apriori coordinates for any stations on the apr_file

based on apr_ and mar_ commands, make a list of parameters to be estimated

initialize the covariance matrix based on the values in the apr_ commands

Forward Kalman filter ( glfor  )  — for each step (h-file),

update the state vector and covariance by applying the process noise

(first epoch may do a direct copy of the h-file covariance if all aprior sigmas > 10 m 			   and no additional parameters—message printed)

read parameter estimates and covariance from h-file

apply corrections to a priori coordinates, orbital parameters, or EOP for any

differences between h-file and apriori (apr_file, svs_file, in_pmu file)

generate appropriate station names based on eq_file  explicit renames and earthquakes

(h-file must start after and end before the time span of the rename)

generate new a priori values of coordinates using EXTENDED option for non-secular motion

check differences between the h-file estimates and the current state vector;

estimate a rotation between the networks implied by the two sets of coordinates, and

if too large, apply the rotation to the h-file estimates

if one or more coordinates differ by too much after rotation, remove the station

remove from the solution (by setting to zero their partial derivatives in the Jacobian) any 				parameter not included by an apr_ command or excluded by the previous step

perform a trial solution by computing the chi-square increment, skipping the h-file if too large

perform the full solution, incrementing the parameter values and covariance matrix

At the end, write the solution vector and covariance matrix into the sol_file (for glorg) and print

file and optionally to an output h-file (out_glb) for another globk run  ( glout )

If a Kalman back solution has been requested (bak_file), repeat the stepping through the h-files

but in reversed order ("forward" solution can be forward or backward in <time> ); same as

forward solution except that the forward and backward estimates are averaged and the solution

is output  (sol_file and print) at each step

Mathematical references:	Herring et al.  [J. Geophys. Res., 95, 12561, 1990]

Dong et al. [J. Geod., 72, 200, 1998]

Using GLOBK, cont.

# What glorg does

Reads the GLOBK com file for controls to read the sol file and loads the loosely constrained solution

Applies constraints to link the adjustments of parameters and/or force them to zero (equate and force commands)

Applies frame constraints in position and/or velocity (separately), estimating translation and/or rotation and/or scale parameters to minimize the adjustment of  position or velocity for the stations in the stab_site list  (iterates to determine weights)

Estimates rotation vectors between stations defining plates (plate and assign_p )

Writes out full Cartesian estimates and, optionally, position and/or velocity in a local frame

+++++++++++++++++++++++++++++++++++++

+ GLORG                 Version 5.08I +

+++++++++++++++++++++++++++++++++++++

FIXA option set: Updating Velocity  of NYA1_GPS from NYAL_GPS

FIXA option set: Updating Velocity  of HARB_GPS from HART_GPS

Stabilization with  50.0% constant,  50.0% site dependent weighting.

Delete sites with  10.0-sigma condition.

Height variance factor    1000.00 Position,   1000.00 Velocity

Min dH sigma Position 0.0050 m;    Min Position RMS 0.0030 m

Min dH sigma Velocity 0.0050 m/yr; Min Velocity RMS 0.0030 m/yr

Velocity system stabilization results

---------------------------------------

X Rotate (mas/yr)     0.10514 +-    0.00603 Iter  4 sum_mgk95_01_021215rs.gdl

Y Rotate (mas/yr)     0.34850 +-    0.00640 Iter  4 sum_mgk95_01_021215rs.gdl

Z Rotate (mas/yr)     0.89303 +-    0.00435 Iter  4 sum_mgk95_01_021215rs.gdl

X Trans    (m/yr)     0.00067 +-    0.00020 Iter  4 sum_mgk95_01_021215rs.gdl

Y Trans    (m/yr)    -0.00386 +-    0.00019 Iter  4 sum_mgk95_01_021215rs.gdl

Z Trans    (m/yr)    -0.00172 +-    0.00013 Iter  4 sum_mgk95_01_021215rs.gdl

Condition Sigmas used     0.0000    0.0000    0.0000    0.0000    0.0000    0.0000

Sites and relative sigmas used in stabilization

VILL_GPS 0.93  KELY_GPS 0.95  STJO_GPS 0.93  BRMU_GPS 0.93  SCH2_GPS 0.98  THU1_GPS 0.95

ALGO_GPS 0.93  RCM5_GPS 2.07  NLIB_GPS 0.93  CHUR_GPS 0.94  DUBO_GPS 0.96  FLIN_GPS 0.94

MDO1_GPS 0.93  PIE1_GPS 0.93  YELL_GPS 0.93  THTI_GPS 1.06  KOKB_GPS 0.93  CHAT_GPS 0.95

KWJ1_GPS 0.95  MARC_GPS 1.08  TRUK_GPS 1.03  VLAD_GPS 0.98  YAKT_GPS 0.96  IRKT_GPS 0.93

KSTU_GPS 0.96  NRIL_GPS 1.00  ARTU_GPS 0.98  ZWEN_GPS 0.93  METS_GPS 0.94  KIRU_GPS 0.94

BOR1_GPS 0.94  GRAZ_GPS 0.94  POTS_GPS 0.93  WTZR_GPS 0.94  ONSA_GPS 0.93  NYAL_GPS 0.97

KOSG_GPS 0.93  BRUS_GPS 0.94

For  114 Velocity Iter  4 Pre RMS    0.0104 m/yr; Post RMS    0.0004 m/yr

Equating parameters:   39 Equates to be applied

#    Sigma    dChi**2   List of parameters

1:  0.00000     1.03 YUZN_GPS ND YSSK_GPS ND

2:  0.00000    13.50 YUZN_GPS ED YSSK_GPS ED

3:  0.00000     5.73 YUZN_GPS UD YSSK_GPS UD

39:  0.00000     0.16 HART_GPS UD HARB_GPS UD

Total change in Chi**2/f is     1.69 for   39 equates, and   39 conditions

Solution chi**2/f now     0.68 with   41169 degrees of freedom

PLATE ROTATION VECTOR RESULTS

-----------------------------

SITES USED TO ESTIMATE POLE POSITIONS

-------------------------------------

PLATE      SITES

EURA_GPS : VILL_GPS VLAD_GPS YAKT_GPS IRKT_GPS KSTU_GPS NRIL_GPS

ARTU_GPS ZWEN_GPS METS_GPS KIRU_GPS BOR1_GPS GRAZ_GPS

POTS_GPS WTZR_GPS ONSA_GPS NYAL_GPS KOSG_GPS BRUS_GPS

NAFD_GPS : KELY_GPS STJO_GPS BRMU_GPS SCH2_GPS THU1_GPS ALGO_GPS

RCM5_GPS NLIB_GPS CHUR_GPS DUBO_GPS FLIN_GPS MDO1_GPS

PIE1_GPS YELL_GPS

PCFC_GPS : THTI_GPS KOKB_GPS CHAT_GPS KWJ1_GPS MARC_GPS TRUK_GPS

SITES ASSIGNED TO PLATE BUT NOT USED IN ESTIMATE

------------------------------------------------

PLATE      SITES

EURA_GPS : FAIR_GPS BILI_GPS _KMS_GPS OSSO_GKA _TIG_GKA UHAZ_GKA

OMS1_GPS TAL1_GPS SEY2_GPS MAG0_GPS SUS1_GPS KUL1_GPS

TOM1_GPS UNR1_GPS OKHA_GPS YSSK_GPS TSKB_GPS TKL1_GPS

KHAJ_GPS VLAD_GPS YAKT_GPS TIXI_GPS DAEJ_GPS SUWN_GPS

XIAN_GPS

TRANSLATION  X (mm/yr)  +-   Y  (mm/yr) +-      Z  (mm/yr) +-    RhoXY   RhoXZ   RhoYZ

-0.30    0.36    0.32   0.40  -0.38   0.25  -0.134 -0.293  0.086  TRAN

PLATE     Wx (deg/My)  +-     Wy (deg/My)  +-    Wz (deg/My)   +-   RhoXY  RhoXZ  RhoYZ

EURA_GPS  0.004364  0.003972  0.000491 0.003257 -0.002378 0.003525  0.117  0.096 -0.063

NAFD_GPS  0.064429  0.003552 -0.061248 0.004752 -0.214216 0.005111  0.044  0.046 -0.417  XYZ

PCFC_GPS -0.058197  0.005374  0.414789 0.002824 -0.816597 0.004482 -0.109  0.091 -0.047  XYZ

PLATE     Lat. (deg)  +-  Long (deg)  +-    Mag (deg/My)  +-    RhoLtLg RhoLtMg RhoLgMg

EURA_GPS  -28.430  43.377   6.419  41.939   0.004994  0.003766  -0.028  0.155  0.099  LLM

NAFD_GPS  -67.462   1.194 -43.550   2.779   0.231928  0.004399  -0.344 -0.410  0.230  LLM

PCFC_GPS  -62.846   0.202  97.987   0.724   0.917751  0.004297   0.177 -0.292  0.171  LLM

PLATE -  PLATE        Wx (deg/my)   +-   Wy (deg/my) +-     Wz (deg/My) +-   RhoXY  RhoXZ  RhoYZ

EURA_GPS-NAFD_GPS -0.060065  0.003725  0.061739  0.004725  0.211838  0.007041 -0.044  0.357 -0.435

EURA_GPS-PCFC_GPS  0.062562  0.007474 -0.414298  0.005692  0.814219  0.006691 -0.033 -0.128 -0.020

NAFD_GPS-PCFC_GPS  0.122626  0.007309 -0.476037  0.006470  0.602381  0.007174  0.039 -0.135 -0.270

PLATE -  PLATE      Lat (deg) +-  Long (deg) +-   Mag (deg/My) +-       RhoLaLg RhoLaMa RhoLgMa

EURA_GPS-NAFD_GPS  67.873  1.485   134.212  2.759   0.228681   0.005783   0.210  0.665  0.064  LLM

EURA_GPS-PCFC_GPS  62.770  0.375   -81.413  1.013   0.915702   0.006497  -0.163  0.122 -0.074  LLM

NAFD_GPS-PCFC_GPS  50.784  0.439   -75.555  0.853   0.777503   0.007597  -0.166  0.060 -0.125  LLM

Checking covariance matrix after equate and force

Global Multi-epochs for velocity

SUMMARY VELOCITY ESTIMATES FROM GLOBK Ver 5.07I

Long.   Lat.     E & N Rate    E & N Adj.     E & N +-    RHO       H Rate   H adj.    +-  SITE

(deg   (deg)      (mm/yr)       (mm/yr)      (mm/yr)                 (mm/yr)

356.048  40.444   0.08 -0.21    0.59   -0.13   0.75  0.74   0.000   -3.23   -3.23   0.90 VILL_GPS*

344.367  27.764  -3.62  0.66  -20.39  -14.73   0.83  0.82   0.002   -1.22   -0.70   0.90 MAS1_GPS

309.055  66.987 -18.72 -1.60   -0.37   -0.33   0.78   0.78  0.000   -4.08   -4.08   1.02 KELY_GPS*

. . .

VEL STATISTICS: For 28 RefSites  WRMS ENU 0.58  0.65  2.15 mm/yr   NRMS ENU  3.40  3.96  4.49

globk/glorg:  parameterization and controls

Treatment of SV orbital parameters  [GLOBK, pp. 39-40, 46-48]

tight, moderate, and loose apr_svs,  mar_svs

linking h-files on a day

Treatment of EOPs (apr and mar)  [GLOBK,  pp. 27-28, 40-41]

Error Models

h-file reweighting / chi-square increments  [GLOBK, pp. 50-51]

white and randow walk noise reweights  sig_neu mar_neu [GLOBK, pp. 46-47]

Frame, center of mass, and plates  [GLOBK, pp. 37-39]

time-variable center of mass Dong et al., Appendix C,  GLOBK, p. 28]

3-D frame stabilization [GLOBK, pp. 33-39]

plate and local frames

global/regional w/o rotation

repeatability vs velocity solutions

Using the log file

bad coordinates

large rotations

global/regional conflict

Handling earthquakes [GLOBK ,  pp. 29-32]

EQ definition and radius of influence

co-seismic constraints

post-seismic constraints

Handling reference monument ties

rename/equate (globk+glorg) [ GLOBK,  p. 31]

supplementary h-files

equates and force – importance of apr values (FIXA command)

Using hfupd and renames to correct height problems [GLOBK, p. 79-80]

File: globk_comb.cmd

* GLOBK command file for daily h-files (repeatability or combination)

*

* This group must be first

eq_file ../tables/eq_renames

make_svs ../tables/sat1.apr

com_file comb.com

srt_file comb.srt

sol_file comb.sol

* Earth-rotation values  (not needed if pmu free in final combination)

in_pmu  ../tables/pmu.bull_b

* File(s) for a priori coordinates

apr_file ../tables/itrf00.apr

* Optionally use separate file for sites to be included and random or Markov noise

x  source ../tables/daily.uselist

x  source ../tables/daily.reweight

* (1) Max chi**2, (2) Max prefit diff, (3) Max rotation; defaults are 100 10000 10000

max_chi 30 50 2000.0

* Apply the pole tide whenever not applied in GAMIT

app_ptid ALL

* Allow the network to be loose since using glorg for stabilization

apr_neu  all  10 10 10  0 0 0

* Satellites are loose if using global data

apr_svs all   100 100 100 10 10 10  1R

* but tight if not combining with global data

x   apr_svs all .05 .05 .05 .005 .005 .005 .01 .01 FR

* EOP loose if estimating rotation in glorg

apr_wob  100 100 10 10

apr_ut1  100  10

* if combining days, possibly add day-to-day noise to absorb rotation-like noise

x  mar_wob    1 1 0 0

x  mar_ut1    1 0

* Set minimal globk print options since using glorg output

prt_opt NOPR

* Invoke glorg for stabilization

org_cmd glorg_comb.cmd

org_opt PSUM GDLF CMDS

* Write out an h-file if needed for future combinations

x  out_glb  H------.GLX

File: daily.uselist

# Start from zero

use_site clear

# Includes sites within NAM and PAC

use_pos +25. 180. 85. 310.

use_site kely thu1

# Remove SCIGN site that existed for only part of 2001 (vandalism)

use_site –SKYB_@

File: eq_renames

* Rename before EQ renames to avoid conflict with SoCal site LAE1

rename LAE1_GPS  LAE1_IGS SIO 1997 1 1 0 0 2100 1 1 0 0

* Earthquake:

*   Izmit

eq_def    IZ  40.702 29.987 300 17.0 1999 08 17 00 00

eq_renam  IZ

eq_post   IZ  90   0.1  0.1   0.1     1.8  1.8  0.7   Post: 0.1 mm**2/day

eq_renam  IZ

# Temporary to fix problem of mistranslated RINEX files in the archive

rename WTM2_GPS WETM_GPS 1989 5 23 0 0 1991 7 24 0 0

rename WETM_GPS WETM_XCL 1991 7 24 0 0 1992 1  1 0 0

# Temporary to fix Hunter 1991 conflict with continuous site from 2001

rename HUNT_GPS HNTR_GPS 1993 1 1 0 0 1997 12 31 24 0

# Bad continuous stations

rename BILL_GPS BILL_XCL 1997 7  1  0  0

rename HARV_GPS HARV_XCL 1992 1 1 0 0 2100 1 1 0 0

* Now apply special renames for antenna changes

*

rename WUHN_GPS WUHN_1PS 2002  1 26  0  0

rename kely_gps kely_1ps 2001  9 14  0  0

File: daily.reweight

#  Downweight verticals with unresolved HI problems

sig_neu @_xhi  0. 0. 1.

sig_neu @_xnr  0. 0. 1.

# High noise in continuous stations (more noise may be needed in velocity run)

sig_neu cit1@ 0 0 .005 1994 1 1 0 0  2100 1 1 0 0

sig_neu chil@ 0 0 .005 1995 1 1 0 0  2100 1 1 0 0

# Outliers in 2003 from Dec 2005 analysis

sig_neu algo@     0.030  0.030  0.000  2003  1 30  0  0  2003  2  9 24  0

sig_neu algo@     0.000  0.000  0.010  2003  9 25  0  0  2003 12 18 24

sig_neu aoml@     0.005  0.005  0.020  2003  1 11  0  0  2003  6 30 24  0

# This one systematic

sig_neu prds@     0.030  0.030  0.030  2003  1 11  0  0  2003  4 18 24  0

File: glorg_comb.cmd      GLORG command file for combination or repeatabilities

* Stabilization using IGS stations

apr_file ../tables/itrf00.apr

* Regional stabilization using coordinates from a prior solution

x  apr_file ../tables/globk_vel_990306.apr

* Define the stabilization frame

source ../tables/stab_site.global

x  source ../tables/stab_site.regional

* Set parameters to estimate in stabilization

pos_org  xrot yrot zrot xtran ytran ztran

* Default height ratio is 10; increase if heights questionable

cnd_hgtv 1000 1000 2.0 10.0

* Default iterations is 4, relative weight ratio 50%,  horizontal sigma cutff 4.0

x  stab_ite 4 0.8 3.

File:  stab_site.global

stab_site clear

stab_site vill fort kour brmu thu1 sant areq algo nlib pie1 yell drao

# omit FAIR after Denali EQ

fair_gps

-fair_gps -fair_gdn R SIO 2002 11 2 0 0

chat auck kwj1 mcm4 mac1 tidb hob2 guam tskb pert yar1 cas1

irkt lhas kit3 bahr mali zwen mate onsa nyal

* added by rwk for 1992 and early 1993

albh graz wett usud usu2 gold_gps -darw kosg stjo

rcm2 ricm wsfm mcm0 well ds41 darw hart hob1 town

x for late 1993 ff

File:  stab_site.regional

* Stabilization list for E Asia repeats from solution vel_031007.org, 1995-2003

*

stab_site clear

stab_site  BILI_GPS  !       0.54    0.52     2.67

stab_site  PETP_GKA  !      0.50    0.47     1.12

stab_site  MAG0_GPS  !     0.79    0.78     1.84

x stab_site  USUD_GPS !      0.23    0.19     0.46

x stab_site  DAEJ_GPS !      0.46    0.41     1.14

stab_site  TAEJ_GPS  !     0.46    0.41     1.14

stab_site  SUWN_GPS  !     0.41    0.33     0.86

stab_site  SHAO_GPS  !     0.40    0.31     0.89

stab_site -shao_gps R CHINA_0109

stab_site  BJFS_GPS  !     0.50    0.46     1.69

x stab_site  WUHN_1PS !      0.71    0.57     3.18

stab_site  WUHN_GPS  !     0.71    0.57     3.18

stab_site  XIAN_GPS  !     1.06    0.74     2.13

x stab_site  TYP1_GPS !      1.20    0.80     2.40

stab_site  TYP2_GPS  !     1.20    0.80     2.40

stab_site  LSC1_GPS  !     0.50    0.39     1.28

x stab_site  RJJ2_GPS !      6.45    1.81     6.25

stab_site  MZZ1_GPS  !     0.83    0.60     1.68

x stab_site  NRIL_GPS !      1.21    1.17     2.91

stab_site  URUM_GPS  !     0.53    0.49     2.20

x stab_site  NVSK_GPS !      1.82    1.62     4.38

stab_site  POL2_GPS  !     0.24    0.20     0.49

stab_site  ARTU_GPS  !     0.46    0.44     2.32

File:  globk_vel.cmd

eq_file ../tables/eq_renames

com_file vel.com

srt_file vel.srt

sol_file vel.sol

* Run the solution backwards in time (default is forward)

x  srt_dir -1

# Earth-rotation values (use only if rot not free in globk and glorg, and daily h-files)

x in_pmu  ../tables/pmu.bull_b

* File(s) of a priori coordinates for combination (may be overrriden by glorg)

apr_file china_040622b.apr

apr_file ../tables/itrf00_noam.apr

* Optionally use separate file for sites to be included and random or Markov noise

x  source ../tables/vel.uselist

x  source ../tables/vel.reweight

* (1) Max chi**2, (2) Max prefit diff, (3) Max rotation; defaults are 100 10000 10000

max_chi 5 2. 20000

* Invoke GLORG and set print options

crt_opt NOPR

prt_opt NOPR

org_cmd glorg_vel.cmd

org_out globk_vel.org

org_opt PSUM CMDS GDLF VSUM FIXA RNRP

* Loose apr for glorg

apr_neu  all 10 10 10  0.2 0.2 0.2

apr_wob  10 10 10 10

apr_ut1  10 10

mar_wob    3650 3650  365 365

mar_ut1    365 365

* Allow translation and scale variations to account for global frame errors

* and SV constellation shift

apr_tran 1 1 1 0.1 0.1 0.1

apr_scale 10 1.

mar_tran   3.65  3.65  3.65     0  0  0

mar_scale  365   0

* Random (sig_neu) and Markov (mar_neu) reweights

source ../tables/vel.reweight

File: vel.reweight

* Random and Markov reweights for velocity solution (on top of comb reweights)

# reweight all monthlies to avoid too-small uncertainties in repeats

sig_neu @ .001 .001 .005

# also add Markov horizontal 1**2 mm/yr, vertical 3**2 mm/yr

mar_neu all .000001 .000001 .000009 0 0 0

# High noise in continuous stations (noise also added in daily combinations)

sig_neu cit1_gnr 0 0 .005 1994 1 1 0 0  2100 1 1 0 0

# Downweight all sites in ETS-affected area by 3 mm to account for

# elevated 'noise' from the slip

sig_neu albh@ .003 .003 0

sig_neu youb@ .003 .003 0

File: glorg_vel.cmd

* GLORG command file for velocities and positions

* Define the stabilization frame

apr_file ../tables/itrf00.apr

source ../tables/ .global

x  source ../tables/ .regional

* Coordinate/velocity files for stabilization and matching apriori values

apr_file 040622a.apr

apr_file ../tables/itrf00.apr

* Set parameters to be estimated (include scale only if estimated in globk)

pos_org xtran ytran ztran xrot yrot zrot scale

rate_org xtran ytran ztran xrot yrot zrot scale

* Default height ratio is 10; increase if heights questionable

cnd_hgtv 1000 1000 2.0 10.0

* Default iterations is 4, relative weight ratio 50%,  horizontal sigma cutff 4.0

x  stab_ite 4 0.8 3.

* Get velocities relative to defined plates (extract using sh_org2vel)

plate NAFD_GPS KELY_GPS STJOS BRMU THU1_GPS ALGO NLIB MDO1 PIE1 YELL

plate PCFC mkea kwj1 chat

* Keep the ITRF00-NNR translation rates when estimating plates

(default is to estimate simultaneously)

x  NOPLATETRAN

* Apply equates

* link velocities of stations within 1 km  (a prioris should match, aided by FIXA)

eq_dist 1000 ndot

eq_dist 1000 edot

eq_dist 1000 udot

* link some more separated by more than 1 km

equate ftor_gps ndot ords_gps ndot for2_gps ndot

equate ftor_gps edot ords_gps edot for2_gps edot

equate ftor_gps udot ords_gps udot for2_gps udot

* unlink some that don't work

unequate wsfd udot

unequate enda_gla ndot

unequate enda_gla edot

unequate enda_gla udot

* link some horizontal positions when heights are uncertain

equate soli_gps npos soli_xhi npos

equate soli_gps epos soli_xhi epos

* link some positions across earthquakes when a model has been applied in the eq_file

source LA_equates

File: glist.out  from running        glist china.gdl china.glist +1 eq_file ‘ ‘ china_050622b.apr

Summary of SITE occurences in china.gdl

Use of sites for  488 sites

soln/CHINA_9911.GLX

99 12  17   2  11  20  22  23  32  35  41  45  47  49  59  61  70  71  74  79  80  83

89  92  96  97 100 103 117 124 125 129 132 140 143 144 152 153 155 160 167 168 169

171 198 209 223 230 237 239 256 258 259 273 283 284 293 301 302 303 312 394 401 408

482 486  -1  -1  -1  -1  -1  -1  -1  -1  -1  -1  -1  -1  1 /raid1/rwk/china99/g

gsoln/CHINA_0306.GLX

103  7 31   1   2   4   7   8   9  10  11  13  14  16  17  18  19  21  22  24  26  28

31  33  34  36  37  38  40  42  43  45  46  50  52  53  54  56  58  59  60  61  63

66  67  68  69  70  71  72  74  75  76  79  80  86  87  89  90  92  93  94  95  96

98  99 102 103 104 106 108 110 112 114 116 117 118 119 124 125 126 129 132 134 137

139 140 142 143 144 147 148 149 151 152 155 160 161 162 163 164 166 167 168 170 173

453 454 455 457 458 459 461 462 464 465 466 467 468 469 470 471 472 474 475 476 477

478 480 481 486   -1   -1   -1   -1   -1   -1   -1   -1   -1  /Hfiles/H2003184_2

SUMMARY of occurences

1. VESL_GPS    4 2003.6-2005.5  1.93      2. VILL_GPS   25 1995.4-2005.5 10.12

3. MADR_GPS    8 1993.4-2005.5 12.11      4. MADR_XCL    1 2003.6-2003.6  0.00

5. MAD2_2PS    3 2005.3-2005.5  0.19      6. SFER_GPS    3 2005.3-2005.5  0.19

...

483. TLSE_GPS    3 2005.3-2005.5  0.19    484. HERS_GPS    6 1995.4-1997.4  1.99

485. HERS_1PS    2 1998.6-1998.6  0.06    486. HERS_XCL   12 1999.9-2005.5  5.60

487. USU3_GPS    0    0.0-   0.0  0.00    488. MDX2_GPS    0    0.0-   0.0  0.00

. SUMMARY of site position and occurences P

.  Long      Lat         Ht     #  First   Last    Dur.  Name     Seq P

.   deg      deg         km                        yrs                P

357.1582 -71.6738   0.8624    4 2003.579 2005.511  1.93 VESL_GPS    1 P

356.0480  40.4436   0.6474   25 1995.396 2005.511 10.12 VILL_GPS    2 P

355.7503  40.4292   0.8295    8 1993.404 2005.511 12.11 MADR_GPS    3 P

355.7503  40.4292   0.8295    1 2003.579 2003.579  0.00 MADR_XCL    4 P

...

0.3363  50.8673   0.0765    6 1995.396 1997.388  1.99 HERS_GPS  451 P

0.3363  50.8673   0.0765    2 1998.563 1998.623  0.06 HERS_1PS  452 P

0.3363  50.8673   0.0765   12 1999.914 2005.511  5.60 HERS_XCL  453 P

There were  323 site renames listed.  Renames used are:

# Orig   New     Specific  Period from   ----->  To    Position change (m)

1 MDX2_GPS->MDX1_GPS    2003/ 3/ 1  0: 0  2003/ 9/ 1  0: 0   0.0000  0.0000  0.0000

10 AREQ_GPS->AREQ_XCL    2001/ 6/22  0: 0  2003/ 4/20  0: 0   0.0000  0.0000  0.0000

There were   14 earthquakes listed.  Earthquakes used are:

#  CODE     Lat (deg)  Long (deg) Radius (km) Depth (km)    Date       Rename?

5   HT      34.5900   243.7300     200.0000     5.0000 1999/10/16 11:46  YES

7   KA      54.3100   161.9100     600.0000    33.6000 1997/12/ 5 11:26  YES

10   AR     -17.2800   287.2900     700.0000    29.6000 2001/ 6/23 20:34  YES

11   KU      35.5400    92.7500    1000.0000    15.0000 2001/11/14  9:26  YES

12   DN      63.7400   212.3100     600.0000    10.0000 2002/11/ 5  0: 3  YES

13   MI     -50.1500   160.3700    2500.0000    20.0000 2004/12/24  1:59  YES

14   SU       4.5000    95.0000    2500.0000    20.0000 2004/12/26  0:57  YES

Use to check:

- correct path/filename in gdl file

-  estimate/apr errors from misidentified sites or bad solutions

-  correct application of renames

Can provide input to program gen_uselist to generate use_site list based on span

# Defining the reference frame

Mathematical methodology [Dong et al., 1998]

finite constraints – GLOBK apr_neu

generalized constraints  – GLORG  , plate

Implementation and geometric considerations [GLOBK pp. 41-43]

global frame

ITRF

NUVEL [Shen et al., J. Geophys. Res., 5721, 2000]

ITRF rotation poles [Altamimi et al., J. Geophys. Res., 2214, 2002]

frames defined by plates or crustal blocks.

[Chen et al., J. Geophys. Res., 105, 16215, 2000]

impact of frame translation on estimated plate rotations

[Steblov et al., Geophys. Res. Lett., 30, 1924, 2003GL017805, 2003]

Analysis of a complex multi-year data set – Southern California 1992-1999

Run sh_gamit for to get daily h-files for each survey; e.g. for 1994:

sh_gamit -expt scec -s 1994 129 138 -pres Y -copt o q mkx -dopt D ao c ps x   >&! sh_gamit.log

--> hsceca.94130, hsceca.94131, ....

Combine the daily h-files from the GAMIT solution with California and global daily h-files generated by SOPAC, and generate time series of daily residuals for inspection (globk_comb.cmd):

sh_glred  -s 1994 129 1994 2001 146  -expt scec  -local  -net igs1 net1  -opt H G E  >! sh_glred.log

Using the time series, remove or downweight outliers:

-add _XCL renames to scec_eq  (also has the Landers EQ defined)

-add  sig_neu downweights to scec_comb.reweight

Aggregate (combine) all of the daily files into 'monthly' or 'survey' H-files  spanning 15-30 days (fewer immediately after the EQ):

sh_glred -s 1994 129 1994 2001 146 -expt scec -ncomb 18 -local -net igs1 net1 -opt H G >! sh_glred.log

--> (e.g.) H940509_SCEC.GLX

Generate a list of H-files for the final GLOBK runs:

- from /gsoln,       ls ../glbf/*.GLX > scec.gdl

The following steps are useful to understand the data content, detect bad aprioris or estimates, and to select the stations to be included in the solution:

Run glist to get a list of all stations and their spans

glist  scec.gdl  scec.glist  +1  scec.eq  scec.apr

The file scec.glist shows that there are 660 stations in the solution to be generated by the H-files in scec.gdl.  To reduce the solution to only those stations that are useful, run the program gen_uselist with different criteria:

For co-seismic displacements, use all stations with at least 2 epochs and a 0.1-yr span

 coseis.uselist  (285 stations)

For reliable velocities, use all stations with at least 3 epochs and a  3.0-yr span

vel.uselist   (69 stations)

Generate time series of long-term behavior, using a plate-scale stabilization

rm globk_rep.org globk_rep.log

glred 6 globk_rep.prt globk_rep.log scec.gdl  globk_rep.cmd  >! glred.out

mv globk_rep.org globk_rep_noam.org

Check for good stabilization on each day:

Grep  'Post RMS'  globk_rep_noam.org  | grep  'Iter  4'  >  rep_rms.noam

Each solution (aggregated h-file) should have least 5 stations in stabilization and rms  < 5 mm.

Plot the time series:

sh_plotcrd -f ../globk_rep_noam.org -s long -x 1992.0 2000.0  -order 1 -res -cols 1

Add downweights (sig_neu) to scec_vel.reweight for any outliers.

Perform a velocity solution:

rm  globk_vel.org  globk_vel.log

globk  6  globk_vel.prt  globk_vel.log  scec.gdl globk_vel.cmd  >!  globk.out

mv  globk_vel.org  globk_vel_051028a.org

Check globk_vel.org (or .log) to make sure all chi2 increments are reasonable are in the range of 0.2-1.5.  If some are higher, look for outliers in the time series and downweight individual stations (sig_neu) or the entire h-file (variance factor in scec.gdl).  Check in globk_vel.org for a good total chi2 (1.25) and a good fit to the reference-frame-defining stations (velocity rms < 2 mm/yr).

Plot the velocities:

sh_plotvel -ps 051028a -f globk_vel_051028a.org -maxsigma 2   -arrow_value 5 -maprange cal_landers

See sh_map_elements for defining 'maprange'; or use

sh_plotvel  -ps 051028a  -f globk_vel_051028a.org  -maxsigma 5 -R-122/-114/31/37.5

Generate a new apr file with a self-consistent set of coordinates and velocities for all stations:

grep 'Unc. ' globk_vel_051028a.org > ../tables/scec_noam.apr     (edit out the 'Unc.' in columns 1-4)

Repeat the generation of time series, this time using a regional stabilization.   In glorg_rep.cmd, replace itrf00_noam.apr with scec_noam.apr, and stab_site.noam with stab_site.calif, where stab_site.calif includes all stations with well-determined positions and velocities. (this might be 3 mm in position for pre-Landers, < 8 mm for post-Landers; < 2 mm/yr in velocity).

rm  globk_rep.org  globk_rep.log

glred  6  globk_rep.prt  globk_rep.log  scec.gdl  globk_rep.cmd  >! glred.out

mv  globk_rep.org  globk_rep_calif.org

Check for good stabilization on each day:

grep  'Post RMS'  globk_rep_calif.org |  grep 'Iter  4'  > rep_rms.calif

When we first ran these repeatabilities with regional stabilization, the surveys had 8-30 stations in stabilization and rms values 1-9 mm.  Most of these were better than for the N-Amer stabilization, but the first 6 surveys after Landers (920708-931005) were significantly worse (3.4-8.2 mm). Examing the glorg print file revealed two obvious problems:

1) We included in the stabilization stations that were affected by (non-linear) post-seismic motion.  We anticipated this problem but included some stations in the hope that the 3-sigma cutoff criterion for the stabilization would force them to be removed in the stabilization iteration (it just barely failed to the outliers).

2) The (many) additional stations in the Landers region reduced the height-sigma mean enough that all but one of the distant (North American) stations were removed by the relative-height-sigma criterion (cnd_hgtv) for the iteration of the stabilization.

We fixed these two problems by removing (commenting out) more Landers-region stations from the stabilization and increasing the height ratio from 2.0 to 4.0.  With these changes, the stabilization rms for the regional frame is only slightly worse than for the N-Amer frame (rep_rms.noam) in 1992.5-1993.8 and better everywhere else.  It is useful at this point to generate time series with the regional stabilization and compare these with the N-American stabilization. It is also useful to perform the regional stabilization with and without rotation estimated in glorg.  Including rotation provides more parameters to absorb orbital and EOP errors, but reduces the redundancy, possibly masking errors associated with stations that have a high weight (“lever arm”) in the stabilization.  The sensitivity of the time series to the spatial extent of the reference frame (stabilization network) and number of parameters estimated increases as the number of stations available for stabilization decreases.

Get the final velocities:

rm globk_vel.org globk_vel.log

globk 6 globk_vel.prt globk_vel.log scec.gdl globk_vel.cmd >! globk.out

mv globk_vel.log globk_vel_051028b.log

mv globk_vel.org globk_vel_051028b.org

(Use sh_plotvel again to get velocity field displayed.)

Repeat the glorg run (only) to get velocities in other frames.  Do this by editing the glorg command file to change stab_site, apr_file, and/or plate.  Add BLEN to the print options to get baselines for co-seismic displacements.

rm globk_vel.org

glorg globk_vel.org CMDS:PSUM:VSUM:GDLF glorg_vel.cmd  scec_vel.com

mv globk_vel.org globk_vel_051028c.org

Extract the co-seismic displacements:

sh_eq   -f globk_vel_051028c.org  -eq GLA

There are two files of co-seismic displacements, in different formats:

eq_disp.globk_vel_021102c.org.out

eq_disp.globk_vel_021102c.org.vel  (prt/org format, for input to sh_plotvel)

# Error analysis and data weighting

From theoretical considerations and empirical evidence

•  phase noise is elevation dependent and has temporal correlations of seconds to hours

•  daily position estimates have temporal correlations of days to months

•  position and velocity estimates have spatial correlations up to ~ 1000 km

therefore using random-noise statistics is almost never reasonable

Approaches to getting reliable statistics

Ideal:  use a full and theoretically complete covariance matrix for the phase analysis and/or

quasi-observation analysis — not possible

Reasonable and often adequate approximation

•	Use a post-processing analysis of the position repeatabilities and external 			knowledge of the noise spectrum to assign the uncertainties on a station-by-			station and component-by-component basis.   Works only for long, continuous

time series.

Utilities: sh_plotcrd / sh_tshist;  ensum / enfit / sh_marcmd

References: 	Langbein and Johnson [J. Geophys. Res., 102, 591, 1997]

Zhang et al. [J. Geophys. Res., 102, 18035, 1997]

Mao et al. [J. Geophys. Res., 104, 2797, 1999]

Dixon et al. [Tectonics , 19, 1, 2000]

Herring [GPS Solutions, 7, 194, 2003]

Williams [J. Geodesy, 76, 483, 2003]

Williams et al. [J. Geophys. Res. 109, B03412, 2004]

•	Decimate and reweight the phase data and/or aggregate and reweight the position 			estimates so that the assumed a priori error for (nearly) random samples gives a chi-square 			of 0.5-1.0 ;  include Markov noise (random walk) in the analysis.  	Often only practical			approach for survey-mode observations.

[McClusky et al., J. Geophys. Res., 105, 5695, 2000; McCaffrey et al., 2006]

•	Use an a posteriori analysis of velocity scatter with respect to an assumed model to assess the reasonableness of the estimated uncertainties.

Utility:  sh_velhist

References:  	McClusky et al., [J. Geophys. Res., 105, 5695, 2000]

Langbein and Johnson [J. Geophys. Res., 102, 591, 1997]

McClusky et al. [Geophys. Res. Lett. 28, 3369, 2001]

McCaffrey et al., [submitted to Geophys. J. Int., 2006].